Solutions of the experimental task

1.	[1 point] Let’s examine the relationship between resistance of the thermoresistor and temperature based on the following setup

Here: 1 – digital thermometer; 2 – distilled water; 3 – thermoresistor; 4 – digital multimeter; 5 – glass.

2.	[3 points] The relationship between resistance R of the thermoresistor and temperature toC

Table №1

| N | t, oC | R, Ohm |
| --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 | 80,0 ± 0,1 75,0 ± 0,1 70,0 ± 0,1 65,0 ± 0,1 60,0 ± 0,1 55,0 ± 0,1 50,0 ± 0,1 45,0 ± 0,1 40,0 ± 0,1 35,0 ± 0,1 30,0 ± 0,1 | 15,6 ± 0,1 17,4 ± 0,1 19,5 ± 0,1 22,0 ± 0,1 25,3 ± 0,1 29,2 ± 0,1 34,3 ± 0,1 40,6 ± 0,1 48,5 ± 0,1 57,3 ± 0,1 71,2 ± 0,1 |

3.	[3 points] The expression

σ=σ0exp−ΔW2kT

сan be linearized, if we take the logarithm of both sides:

lnσ=lnσ0−ΔW2kT

Therefore

lnσ=−ΔW2k1T+lnσ0

4.	[2 points] The values that we need are as follows:

Table №2

| N | t,oC | 1/T, K-1 | R, Оhm | σ = 1/R, Ohm-1 | ℓn (σ Ohm) |
| --- | --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 | 80,0 75,0 70,0 65,0 60,0 55,0 50,0 45,0 40,0 35,0 30,0 | 0,00283 0,00287 0,00292 0,00296 0,00300 0,00304 0,00309 0,00314 0,00319 0,00324 0,00330 | 15,6 17,4 19,5 22,0 25,3 29,2 34,3 40,6 48,5 57,3 71,2 | 0,0641 0,0575 0,0513 0,0455 0,0395 0,0342 0,0292 0,0246 0,0206 0,0175 0,0140 | – 2,75 – 2,86 – 2,97 – 3,09 – 3,23 – 3,38 – 3,53 – 3,71 – 3,88 – 4,05 – 4,27 |

5.	[3 points] Below is the graph of ℓn σ  versus  1/Т, using ordinary least squares:

6.	[3 points] According to the result in 5)

a)	The slope of the linear relationship is 3,27∙103K. Then

ΔW2k=3,27∙103K

Then the width of the forbidden zone is

∆W = 3,27∙103∙2∙k = 3,27∙103∙2∙1,38∙10-23J = 0,90∙10-19 J = 0,56 eV

The uncertainty is

(∆W) = 0,06∙103∙2∙k = 0,06∙103∙2∙1,38∙10-23 = 0,02∙10-19 J = 0,01 eV

- свободный член уравнении равняется 6,6. Тогда

ln(σ0Ом)=6,6

и значение σ0 равняется

σ0=1,87 Ом-1

Соответственно погрешность

Δσ0=0,37 Ом-1

Finally

ΔW = (0,56±0,01) eV

σ0=(1,87±0,37) Ом-1